Normal Approach and Landing (KIAS)

Verify pattern altitude and weather no later than 10 nm from airport.

45 degree entry

Begin slowing to 90 KIAS and plan decent to enter the pattern.

Downwind

Abeam touchdown point complete before landing checklist, add carb heat, reduce power to 1500 rpm, set flaps 10, descend at 80 KIAS.

Base

Final

Turn base 45 degree angle from touchdown point.

Select flaps 20 and slow to 70 KIAS.

Select flaps 30 and slow to 65 KIAS.

Aiming Point

Touchdown Point

90 KIAS Approx 2100 RPM