- Notes box

- Aircraft tail number

- Departure and Arrival airports

- Selected altitude

- Notes box

- Gather forecasted weather for the departure airport and the arrival airport

- Gather winds aloft for the altitude you're flying

- Notes box

- Calculate Takeoff, Cruise, and Landing Pressure Altitude

- (29.92 - altimeter setting) 1000 + Field elevation or Cruise altitude = PA

- Notes box

- Calculate Climb Leg (Time, Fuel, Distance to Climb Chart)

- Calculate Cruise (Cruise Chart)

- Calculate Range (Range Chart)

- Checkpoints/Fixes box

- First two boxes put departure airport and Top of Climb (TOC)

- Pick visual points (10-15 NM apart)

- Last box put arrival airport

- Distance box

- First box place distance you got from (Time, Fuel, Distance to Climb Chart)

- Use foreflight measure tape for the other boxes/ Plotter

- Last box use the 3 to 1 rule to calculate distance to descend

- Altitude box

- First box put climb leg

- Other boxes place cruise altitude selected

- Last box put decent leg

- Course box

- Find True Course using ruler function

- If the ruler is in magnetic got to setting

- Go to the bottom where “Units/Time

- “Bearing and Track Instrument” select “True”

- Or use plotter and paper sectional

- Wind box

- First box put winds at the departure airport

- Others put wind from winds aloft

- Last box put wind at the arrival airport

- TAS Box

- First box find IAS from ((Time, Fuel, Distance to Climb Chart)

- Then go to (Airspeed Calibration Chart) (Flaps up section)

- Find CAS

- The go to electronic e6b (Planned true airspeed)

- Put calculated TAS in first box

- Other boxes put TAS from (Cruise Chart)

- Last box we descend at 90 KIAS

- Then go to (Airspeed Calibration Chart) (Flaps up section)

- Find CAS

- The go to electronic e6b (Planned true airspeed)

- True Course and True Heading boxes and Groundspeed boxes

- Place True course in the top box below (TC)

- Finding WCA (Wind Correction Angle) use electronic e6b (Heading and Ground Speed

- From calculation put WCA in the bottom box and then add or subtract from True course number and place the calculated number in the top box under TH

- Then put Ground Speed in (GS) box

- Repeat for the other boxes

- Magnetic Heading box

- Find nearest isogonic line to your route

- West is best (add) and East is least (Subtract)

- Add or Subtract from TH and place the new calculation under the MH top box

- ETE and ETA box

- Use electronic e6b (Leg Time)

- Put leg times in each box

- Fuel box

- Use electronic e6b (Fuel required)

- Put fuel in gallons in each box

- Airport frequency box

- Write down departure and Arrival frequencies